A Phycoremediation approach for the treatment of Dairy effluent

Water is the most ubiquitous material in nature and is the most vital for increasing industrialization, urbanization and developmental activities to cope up with the population explosion (Rao et al., 1998). Industrialization is linked to economic growth and human progress. However, increase in the number of industrial installation is very much associated with a substantial increase in the quantum of the liquid waste. This treated effluent is being indiscriminately discharged, either directly to the open land or into the nearby water body, which in turn becomes the source of water, required to irrigate the crops. Though, the soil is naturally characterized with a great degree of filtration and decomposition capabilities, the treated discharge effluents containing huge amount of nutrients which may be increase or retained by soil as pollutant.  Therefore, it is essential to control or mitigate these organic and inorganic soil contaminants so as to prevent them from polluting the surface and groundwater by dissolution or dispersion (McLaughlin et al., 2000).

One of the most commonly employed method of effluent treatment is the 	microalgae based innovative phycoremediation technology. It is eco-friendly and in 	an alternative to conventional treatment methods are gaining momentum. 	Phycoremediation can be considered as one of the most inexpensive method of 	pollution cleanup. Development of these methods are highly attractive on the 	corporate front due to high price tags associated with conventional remedial measures. 	To develop suitable and efficient treatment systems, it is obligatory to understand the 	mutual influence and interactions between the effluents and the organisms, so that 	manipulations to improve the treatment systems become feasible.

When considering biological wastewater treatment for a removal of specific 	nutrients, it is important to understand the sources of the wastewater generated, 	wastewater composition, discharge requirements, events and practices within a 	facility that can affect the quantity and quality of the wastewater and pretreatment 	ramifications. Consideration of these factors will allow us to maximize the gain from 	effective biological treatment.

Microalgae and Wastewater Treatment

The role of algae in the removal of various kinds of inorganic and related 	substances through bioremediation has been studied by several workers during the last 	several years. The ability of cyanobacteria (blue green algae) to reduce pollution load 	in industrial wastewater has been studied by various authors all over the world 	(Oswald, 1963; Senegar and Sharma, 1981; Uma and Subramanian, 1990; Manoharan 	and Subramanian, 1992; Olguın et al., 1994; Kotteswari et al., 2007; 2012; 	Murugesan et al., 2007; 2012). Blue green algae are ideally suitable for the treatment 	of wastewater and in the process of effective utilization of different constituents 	essential for growth leading to enhanced biomass production (Rawat et al.,2011).  	Algal growth rate controls effectively directly and indirectly the nitrogen and 	phosphorus removal efficiency. Thus, maximum algae productivity is required for the 	pollutant removal and must be considered as a key area of research. Likewise, low 	harvesting costs of algae required for a cost-effective nutrient removal system.

Present Investigation

Dairy effluent is one of the significant contributors to ground water pollution. 	This will disturb the normal functioning of cells in fauna and flora due to the 	chemicals present in the waste and this in turn, may result in the alternation, in the 	fundamental biochemical and metabolism of animals (Sin, 1990; Maheswari Devi 	et al., 1991). In the present study, dairy effluent was collected from Central Dairy 	Plant (Aavin), Madhavaram, Tamilnadu, India at regular interval from the point of 	intake to treatment plant and to discharge point of the Effluent Treatment Plant 	(E.T.P) outside the premises.

The principle objectives were covered both in vitro and in vivo experiments

  - Isolation, purification and identification of the naturally occurring dominant cyanobacteria from the effluent Central Dairy Plant (Aavin), Madhavaram, laboratory maintained cyanobacteria with high biodegradation and/or removal capabilities for organic pollutants.

  - Quantification of physico-chemical parameters of untreated and treated dairy effluent.

  - To find out the tolerance of cyanobacteria to evaluate the efficiency of algae for best possible removal of nutrients, organic pollutants, and heavy metals from the dairy 	effluent.

  - To evaluate the Environmental Impact Assessment studies.

  - To evaluate the value added products from algal biomass.

  - Statistical analysis to check the significance of variance of the observed data and correlated.

  - Policy and managing dairy effluent.

Feasibility studies at in vitro condition

In the present study, the cyanobacterial population collected from the effluent 	were cultured in the Algal Culture Laboratory, Department of Botany, Pachaiyappa’s 	College, Chennai-30. The pure cultures of cyanobacteria were maintained in BG 11 	medium (Rippka et al.,1979) and CFTRI medium (Venkatraman et al., 1985).  The 	following genera of cyanobacteria Anabaena, Nostoc, Oscillatoria, Spirulina, 	Chroococcus, Chroococcidiopsis, Aulosira, Tolypothrix, Phormidium, Stigonema, 	Scytonema, Fischerella, Cylindrospermum, Hapalosiphon and Calothrix were 	cultured. The above mentioned or listed algae were inoculated in dairy effluent, 	individually as well as a consortium.  The physico-chemical parameters, heavy metals 	and bacteriological examination were analyzed on the 15th day according to standard 	procedure (APHA, 2005).

Experiment at in vivo

Phycoremediation is an alternative cost effective, efficient, and sustainable 	technology. In the present study field experiments were carried out to explore the 	potentials of cyanobacteria in phycoremediation of dairy effluent. By using the data 	generated, effective phycoremediation and management of pollution of water 	ecosystems can be put into practice.

Totally 15 species of cyanobacteria (including heterogenous and non-	heterogenous) belonging to 15 genera falling under 6 families were identified. Four 	different genera of bacteria were isolated and identified. Among the cyanobacteria 	isolated, Aulosira and Tolypothrix sp were further selected to treat the effluent. In the 	Dairy effluent, Aulosira and Tolypothrix sp were found to be the most dominating 	genera.  Therefore, it is concluded that cyanobacteria Aulosira and Tolypothrix sp 	could be potentially employed for the treatment of dairy effluent.

Value Added Products

Cyanobacteria have vast industrial and economic potential as valuable sources 	for health foods, carotenoids, dyes, fine chemicals, biofuels and others. The biomass 	of both untreated and treated cyanobacteria was analyzed for the various biochemical 	parameters for pharmaceuticals, nutraceuticals and other value added products. The 	results were analyzed by IR spectrum and HPLC methods and the results obtained 	were interpreted and discussed. The results of the present study showed a significant 	variation in quality and quantity in chlorophyll, carotenoids, total carbohydrate, 	protein, amino acids, lipids, vitamins and mineral contents in algal biomass with 	respect to control. The analyses of results obtained are presented in detail in the thesis. 	Similarly, antioxidant potentials and antibacterial activity of untreated and treated 	Cyanobacterial biomass were also analyzed and the results were also discussed.

Environmental Impact Assessment

Any effluent treatment method should be safe to the environment and hence 	the environmental safety credentials of Aulosira and Tolypothrix were assessed. In 	this study, Aulosira and Tolypothrix were successfully used for seed germination and 	also as a feed for fishes. The cyanobacteria supported growth more than that of 	control and hence the algae could be used as biofertilizers.

Hence, the present study provide us an alternative technology for a cost-	effective, most importantly, a safer method of cleaning an aqueous pollution. 	Employing this technology in the treatment of industrial effluents presents an 	alternative tool to the current practice of using conventional methods. This study has 	proved that the cyanobacteria can be used as an efficient phycoremediation agent as it 	brings out reduction of pollutants like organic, inorganic compounds and heavy 	metals. The system, when standardized would not only be economical but also eco-	friendly and sustainable.

References

APHA. 2005. “Standard methods for the examination of water and wastewater.” 	American Public Health Association, American Water Works Association, and the 	Water Environment Federation.

CFTRI, 1985. Venkatraman, L.V. and  Becker.E.W. In Biotechnology and Utilization 	of Algae Indian Experience. Department of Science and Technology. New 	Delhi.Cluis, C.2004."Junk-greedy greens: Phytoremediation a new option for solid 	decontamination."BioTech Journal, 2, 6l-67.

Kotteswari, M., Murugesan S. and Ranjith Kumar R.2012.Phycoremediation of Dairy 	Effluent by using the Microalgae Nostoc sp. Int. Journal of Environ. Research and 	Development. 2 (1): 35-43.

Kotteswari, M, Murugesan, S, Kamaleswari, J and Veeralakshmi, M. 2007. 	Biomanagement of Dairy effluent by using Cyanobacterium. Indian Hydrobiol. 10 	(1): 109-116.

Maheswari Devi.K, Gopal V and Rathna Gopal.1991. Liver Somatic index of Channa 	stratus as a biomonitoring tool heavy mental and pesticide toxicity. J. Ecotoxicol. 	Environ. Monit. 1: 135-141.

Manoharan, C. and G. Subramanaian. 1992. Interaction between paper mill effluent 	and the cyanobacterium Oscillatoria pseudogeminata var. Unigranulata. Poll.Res.11 	(2): 73-84.

McLaughlin, M.J. Zarcinas, B.A., Stevens, DP and Cook, N. 2000. ‘Soil testing for 	heavy metals”. Commun Soil Plant Anat.31, 1661-1700.

Murugesan, S. Dhamotharan, R. Ranjith Kumar, R. Mutanda, T, Bux. F. 2012.  	Phycoremediation: An Alternative for Cost-Effective Treatment of Waste Streams.

Advances in Environ Res 24. Nova Science Publishers. Newyork. 268.

Murugesan, S, Dhamotharan, R and Kamaleswari, J .2007.  Phycoremediation of oil 	refinery effluent using cyanobacterium. Eco.Env & Conv. 13(4): 35-40.

Olguı´n, E.J., Herna´ndez, B., Araus, A., Ramı´rez, M.E., Camacho, R., & Galicia, S.  1994. Simultaneous high biomass protein production and nutrient removal using Spirulina maxima in seawater supplemented with anaerobic effluent. World J Microbiol   Biotech, 10, 5, 576 – 578.

Oswald, W.J.1963. High rate ponds in waste disposal. Dev Ind Microbiol, 4, 112– 9.

Rao, L.M., Rameshwari.Y.1998. Incidence of heavy metal pollution in Mehadrigedda 	stream of Vishakhapatnam. Poll. Res. 17(2): 153-155.

Rawat, I., Ranjith Kumar, R., Mutanda, T and Bux, F. 2011. Dual role of microalgae:

Phycoremediation of domestic wastewater and biomass production for sustainable biofuel production. Appl. Energy. 88, 3411–3424.

Rippka, R., J. Deruelles, J. Waterbury, M. Herdman and R. Stanier. 1979. Generic 	assignments, strain histories and properties of pure cultures of cyanobacteria. J. Gen. 	Microbiol. 111: 1-61.

Senegar, R.M.S., K.D. Sharma and S, Mittal. 1981. In vitro studies for the publication of river water by algal treatment. Geobios. 17: 77-81.

Sin, Y.M. Jch W. F. young MK and Reddy P.K 1990. Effects of mercury on gluthrone thyroid hormones. Bull Environ contain Toxicol 44:616-622.

Uma, L. and G. Subramanian. 1990. Effective use of cyanobacteria in effluent treatment. National symposium on Cyanobacterial in nitrogen fixation. New Delhi. 437-443.